# Bit16 Manual

Contents

Bit16 CPU Layout	3

Bit16 Specifications	4

Instructions and Cycle Times	4

Registers	4

RAM Module	5

Format Summary	5

Microcode Summary	6

ALU Opcode Summary	6

Load Operation Summary	6

Jump Instructions Summary	7

Macrocode Summary	7

Micro Instructions Formats	8

Format 0: Jump	8

Operation	8

Examples	8

Format 1: Binary ALU Operation	9

Operation	9

Examples	9

Format 1: Unary ALU Operation	9

Operation	9

Examples	9

Format 1: Binary Operation with Immediate	10

Operation	10

Examples	10

Format 2: Byte Operation	11

Operation	11

Examples	11

Format 3: Register Offset	12

Operation	12

Examples	12

Format 3: Frame Pointer Offset	12

Operation	12

Examples	12

Format 4: Load/Store	13

Operation	13

Examples	13

Format 4: Load/Store from Frame Pointer	14

Operation	14

Examples	14

Format 5: Load/Store with Register Offset	15

Operation	15

Examples	15

Format 6: Stack Operation (Push/Pop)	16

Operation	16

Examples	16

Format 7: Load Word	17

Operation	17

Examples	17

# Bit16 CPU Layout

Figure 1: Layout of the bit CPU

# Bit16 Specifications

## Instructions and Cycle Times

Each instruction is 16 bits long. More information on the formats of each type of 16-bit instruction can be found later in this document. Every microcode instruction has a clock cycle of 1 except for the load word instruction which takes 2 clock cycles.

## Registers

The bit16 CPU contains 8 addressable 16-bit registers: 4 general-purpose registers and 4 special purpose registers. The general-purpose registers can be used for anything including arithmetic and address calculations. There is also a 16-bit instruction register that stores the executing instruction and a 4-bit flag register which stores the different flags from the ALU’s operations.

The stack pointer (SP) register is used to keep track of the top of the stack. The stack starts at the last address in RAM and grows downward.

The frame pointer (FP) points to the beginning of the current stack frame and is used to load and store local variables on the stack.

The link register (LR) is used for calling and returning from procedures. LR is 16 bits and should only be used to calculate addresses for the RAM module.

The program counter (PC) register keeps track of the current instruction being executed. The program counter starts at 0 and is incremented after every instruction unless the instruction is a jump instruction in which case, the value in the PC becomes the target for the jump.

| Code | Mnemonic | Purpose | Bits | Addressable |
| --- | --- | --- | --- | --- |
| 0 | A | General purpose | 16 | a |
| 1 | B | General purpose | 16 | a |
| 2 | C | General purpose | 16 | a |
| 3 | D | General purpose | 16 | a |
| 4 | FP | Frame pointer | 16 | a |
| 5 | SP | Stack pointer | 16 | a |
| 6 | LR | Link register | 16 | a |
| 7 | PC | Program counter | 16 | a |
|  | Flag | Flag register | 4 |  |
|  | IR | Instruction register | 16 |  |

Table 1: bit16 CPU registers

## RAM Module

The Random-Access-Memory (RAM) module is addressable in the first half of the address space. This allows for 32,768 16-bit data/instruction locations. This makes the RAM module roughly 64 kilobytes in size.

Figure 2: Random-access-memory module

# Format Summary

The bit16 instruction set formats are shown in the following figure.

Figure 3: bit16 instruction set formats

# Microcode Summary

The following section summarizes the bit16 instruction set microcode.

## ALU Opcode Summary

The following table summarizes the bit16 instruction set operations.

| Op code | Mnemonic | Instruction | Condition codes set |
| --- | --- | --- | --- |
| 0 | MOV | Move to register |  |
| 1 | ADD | Add |  |
| 2 | SUB | Subtract |  |
| 3 | CMP | Compare | a |
| 0 (4) | NOT | Bitwise not |  |
| 1 (5) | NEG | Negative |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 | MUL | Multiply |  |
| 9 | DIV | Divide |  |
| 10 | MOD | Modulo |  |
| 11 | AND | Bitwise and |  |
| 12 | OR | Bitwise or |  |
| 13 | XOR | Bitwise exclusive or |  |
| 14 | SHR | Shift right |  |
| 15 | SHL | Shift left |  |

Table 2: bit16 instruction set opcodes

## Load Operation Summary

The following table summarizes the Load operation for the bit16 instruction set.

| Mnemonic | Instruction |
| --- | --- |
| LD | Load |
| ST | Store |
| LDW | Load word |

Table 3: bit16 load instruction

## Jump Instructions Summary

The following table summarizes the jump codes for the bit16 instruction set.

| Code | Mnemonic | Instruction |
| --- | --- | --- |
| 0 | JNV | Jump never |
| 1 | JEQ | Jump if equal |
| 2 | JNE | Jump if not equal |
| 3 | JMI | Jump if minus |
| 4 | JPL | Jump if plus |
| 5 | JVS | Jump if overflow set |
| 6 | JVC | Jump if overflow clear |
| 7 | JCS/JHS | Jump if carry set/higher or same |
| 8 | JCC/JLO | Jump if carry clear/lower |
| 9 | JHI | Jump if higher |
| 10 | JLS | Jump if lower or same |
| 11 | JLT | Jump if greater than |
| 12 | JGE | Jump if less than |
| 13 | JLE | Jump if greater than or equal to |
| 14 | JGT | Jump if less than or equal to |
| 15 | JR/JMP | Jump relative |

Table 4: bit16 jump instructions

# Macrocode Summary

The following section summarizes the bit16 instruction set macrocode. Macrocode instructions are instructions that are not supported by the hardware and are made up of one or more microcode instructions.

| Mnemonic | Instruction | Microcode |
| --- | --- | --- |
| PUSH | Push register(s) to stack | PUSH A, B = PUSH A PUSH B |
| POP | Pop register(s) from stack | POP A, B = POP B POP A |
| JUMP | Jump to immediate address | JUMP L = LDW PC, L |
| CALL | Call procedure | CALL L = ADD LR, PC, 3 JUMP L |
| RET | Return from procedure | RET = MOV PC, LR |
| HALT | Halt execution | HALT = MOV PC, PC |

Table 5: bit16 macro code instructions

# Micro Instructions Formats

The following section covers the 8 different instruction formats of the bit16 CPU instruction set.

## Format 0: Jump

Cond: Condition code

Offset9: Immediate signed 9-bit offset value

Figure 4: Format 0

### Operation

Instructions of this format perform a conditional relative jump. This means that the given immediate value is added to the program counter depending on the state of the flag register.

### Examples

JR	loop	; Unconditionally jumps to the label “loop”

JNE	.L0	; Jumps to the label “.L0” if the Z bit is clear

JGE	end	; Jumps to label “end” if greater than or equal to

## Format 1: Binary ALU Operation

Rd: Source/destination register

Rs: Source register

Op4: 4-bit opcode

Figure 5: Format 1

### Operation

Instructions of this format perform a binary ALU operation on the given registers and place the result in the destination register (Rd) unless it’s a comparison operation (CMP, CMN, TST, TEQ).

### Examples

ADD	A, B	; A = A + B

CMP	A, B	; Subtract B from A and set the condition codes

OR	A, B	; A = A | B

AND	A, B	; A = A & B

MOV	A, B	; Move the value in B to A

## Format 1: Unary ALU Operation

Rd: Source/destination register

Op1: 1-bit opcode. (0 = NOT, 1 = NEG)

Figure 6: Format 1 (Unary)

### Operation

Instructions of this format perform a unary ALU operation on the given source register and place the result in the destination register.

### Examples

NOT	A	; A = ~A

NEG	A	; A = -A

## Format 1: Binary Operation with Immediate

Rd: Source/destination register

Const5: Immediate signed 5-bit value

Op4: 4-bit opcode

Figure 7: Format 1 w/ immediate

### Operation

Instructions of this format perform a binary ALU operation on the given register and the given immediate value and place the result in the destination register (Rd) unless it’s a the comparison operation

### Examples

ADD	SP, 1		; SP = SP + 1

CMP	B,  0		; Subtract 0 from B and set the condition codes

AND	A,  0b111 	; A = A & 0b111

MOV	A,  -1	; Move the value -1 to A

## Format 2: Byte Operation

Rd: Destination register

Const8: Immediate 8-bit value

Op2: 2-bit opcode (0 = MOV, 1 = ADD,

2 = SUB, 3 = CMP)

Figure 8: Format 2

### Operation

Instructions of this format load one 8-bit value into one of the addressable registers.

### Examples

CMP	A, ‘\0’	; Compares the ascii value in A to ‘\0’ (0)

MOV	B, ‘\n’	; Loads the ascii value for ‘\n’ (10) into register B

ADD	B, ‘A’	; Adds the ascii value for ‘A’ (65) to register B

## Format 3: Register Offset

Rd: Source/destination register

Rb: Base Register

Const5: Immediate signed 5-bit value

Figure 9: Format 3

### Operation

Instructions of this format add or subtract a 5-bit signed immediate value to or from the given base register and place the result in the destination register.

### Examples

ADD	A, B, 0x1	; A = B + 0x1

SUB	A, B, 3	; A = B - 3

ADD	A, A, 1 	; A = A – 1. Equivalent to ADD A, 1

## Format 3: Frame Pointer Offset

Rd: Source/destination register

Const8: Immediate unsigned 8-bit value

Figure 10: Format 3

### Operation

Instructions of this format add or subtract an 8-bit unsigned immediate value to or from the frame pointer register and place the result in the destination register.

### Examples

ADD	A, FP, 1		; A = FP + 1

ADD	B, FP, 0xFF		; B = FP + 0xFF

## Format 4: Load/Store

Rd: Source/destination register

Rb: Base register

Offset5: Immediate signed 5-bit offset

S: Store flag (0 = Load, 1 = Store)

Figure 11: Format 4 with immediate offset

### Operation

Instructions of this format transfer 16-bit values to and from registers and the RAM module. The load or store operation depends on the S flag. The offset value is added to the value in the base register and sent to the RAM module’s address BUS.

### Examples

ST	[SP, 2], C	; Stores the value in C to the address SP + 2

LD	C, [SP, 2]	; Loads the value at address SP + 2 into C

LD	A, [A]	; Loads the value at the address in A into A

ST	[B], A	; Stores the value in A to the address in B

## Format 4: Load/Store from Frame Pointer

Rd: Source/destination register

Const8: Immediate unsigned 8-bit value

S: Store flag (0 = Load, 1 = Store)

Figure 12: Format 4

### Operation

Instructions of this format transfer 16-bit word values to and from registers and the RAM module. The load or store operation depends on the S flag. The offset value is added to the value in the frame pointer register and sent to the RAM module’s address BUS.

### Examples

ST	[FP, 0], A		; Stores the value in A to the address FP + 0

LD	A, [FP, 4]		; Loads the value at address FP + 4 into A

LD	A, [FP, 0xFF]	; Loads the value at address FP + 0xFF into A

## Format 5: Load/Store with Register Offset

Rd: Source/destination register

Rb: Base register

Ro: Offset register

S: Store flag (0 = Load, 1 = Store)

Figure 13: Format 5

### Operation

Instructions of this format transfer 16-bit values to and from registers and the RAM module. The load or store operation depends on the S flag. The value in the offset register is added to the value in the base register and sent to the RAM module’s address BUS.

### Examples

ST	[B, C], A	; Stores the value in A to the address B + C

LD	A, [B, C]	; Loads the value at address B + C into A

## Format 6: Stack Operation (Push/Pop)

Rd: Source/destination register

P: Push/pop flag (0 = Pop, 1 = Push)

Figure 14: Format 6

### Operation

Instructions of this format transfer 16-bit values to and from registers and the stack in the RAM module. The push or pop operation depends on the P flag. The SP register is also altered depending the operation.

### Examples

PUSH 	A	; Pushes A to the stack and decrements SP

POP	A	; Pops the top of the stack into A and increments SP

## Format 7: Load Word

Rd: Source/destination register

Figure 15: Format 7

### Operation

Instructions of this format load a full 16-bit immediate value into one of the addressable registers. The 16-bit value is stored in the next location in memory.

### Examples

LDW	A, 3000	; Loads the value 3000 from memory into register A

LDW	B, =msg	; Loads the label address into register B

LDW	C, 0x7fff	; Loads the hex value 7fff into register C